OWNERS MANUAL

MODEL:

STREET VENDOR 4
Professional Popcorn Machine
PART NUMBER:

12040

230 volts, 60 hz


[image: image8.png]Be Careful
e



 WARNING: Handling the cord on this product may expose you to lead, a chemical known to the State Of California to cause cancer and birth defects or other reproductive harm. WASH HANDS AFTER HANDLING. 
1. Electrical ground is required on this appliance.

2. Do not modify the power supply cord plug. If it does not fit the outlet, have a proper outlet installed by a qualified electrician.

3. Do not use an extension cord with this appliance.

4. Check with a qualified electrician if you are in doubt as to whether the appliance is properly grounded.

SAFETY INSTRUCTIONS

1.      Plug electrical cord into a certified electrical outlet with a corresponding voltage rating as listed on the machine model labels.

2.      Always unplug the electric cord when appliance is not in use and before cleaning, adjusting or maintaining this steamer. Do not operate with a damaged cord or plug or after the machine malfunctions, has been dropped or damaged. Use authorized service for examination, electrical or mechanical adjustment or repair.

3       This equipment will be HOT during use and for a short time after it is turned off  BE CAREFUL. To avoid burns, do not let bare skin touch hot surfaces. If provided, use handles when moving this appliance. Keep combustible materials away from appliance.

4.      Extreme caution is necessary when any appliance is used by or near children and whenever the appliance is left operating and unattended.

5.      This appliance is not intended for use in wet locations. Never locate appliance where it may fall into a water container.

         Do not insert or allow foreign objects to enter any ventilation or exhaust opening as this may cause an electrical shock, fire or damage to the appliance.

[image: image2.jpg]



Benchmark USA Inc. ● 25-J Brookfield Oaks Dr. ● Greenville, SC 29607 ● T: 864-312-5500 ● F: 864-312-5505
OPERATING INSTRUCTIONS

1. Remove the top cover of the popper by taking out the four screws located in each corner.  Place the decorative cornice on either the front or back of the canopy and secure using the supplied three washers and nuts.  Replace the top cover.

2. Plug the machine into a properly grounded 120 volt power supply.

3. Install a 50 watt or smaller light bulb in the socket in the chassis (not necessary for operation).  For best results use a 50 watt R20 mini flood bulb.

4. Turn on the “MASTER” switch and the interior lamp will come on and the warming element in the base will be activated.

5. Turn on the “KETTLE” switch and the heat to the kettle and the agitating motor will be activated.  Wait approximately 5 minutes to allow the kettle to get up to operating temperature.

6. Place ½ cup of raw popcorn kernels, 1/6 cup of cooking oil and 1 teaspoon of seasoning salt into the kettle and lower the kettle lid.  (use Benchmark USA part # 42004 – Popcorn measure kit)
7. Allow the corn to pop until the popping ceases (3 seconds between pops) then dump the kettle by turning the handle clock-wise and pulling downward.
8. For additional batches repeat steps 5 & 6.  It will not be necessary to wait 5 minutes since the kettle will already be up to operating temperature.
9. After popping the final batch, turn off the “KETTLE” switch and with a damp cloth wipe any excess oil from the inside and outside of the kettle to prevent burned on oil stains.  Use caution as the kettle will be HOT!
10. If you leave the “MASTER” switch in the on position, the warming element in the base will keep any popped corn fresh and warm.  The interior lamp will remain on as well and serve as a reminder that the popper is on.
KETTLE CARE
CAUTION:  NEVER SUBMERGE THE KETTLE IN ANY LIQUID!  IT WILL SHORT OUT THE ELECTRONIC COMPONENTS INSIDE OF IT AND VOID THE WARRANTY!
1. Always turn off the “KETTLE” switch after popping the final batch of a session.  Leaving it on will burn any residue oil that is left in the kettle.
2. For routine cleaning: After popping the final batch of a session, use a damp cloth to wipe the excess oil from the interior and exterior of the kettle.
3. Benchmark USA kettles are anodized to create an easy cleaning, non-stick surface.  However, if excess oil is not removed it will eventually carbonize on the surface.  For heavily built-up carbon removal it is recommended to use (Benchmark USA part # 43001 – kettle cleaning kit).
CABINET CARE
Always unplug the machine from the power supply before cleaning!

1.    The Stainless steel bottom tray, old-maid drawer and drop door are removable for easy cleaning in a sink or dishwasher.  Use a mild detergent and water.

2. Use a standard glass cleaner for the glass panels and corner posts.

Page 2
[image: image3.png]POPPER ASSEMBLY

(complete part list on page 7)

4008

4002
4002

5026

1011, 1021
1012

4020

4008 iz L B - ———e—

4006 <



[image: image4.emf]4 OUNCE KETTLE ASSEMBLY
(complete parts list on page 7) 
[image: image5.png]Plug Wire Detail





Page 5

[image: image6.png]CHASSIS ASSEMBLY

(complete part list on page 7)

2018,2028

5004,5006,5008

5014,5016,5018




Page 6

PARTS LIST

	P/N
	Description
	
	P/N
	Description

	0014
	4 ounce kettle -120 volt
	
	4001
	Side glass

	0024
	4 ounce kettle -240 volt
	
	4002
	Front glass

	0016
	6 ounce kettle -120 volt
	
	4003
	Glass channel set

	0026
	6 ounce kettle -240 volt
	
	4004
	Black canopy top

	0018
	8 ounce kettle -120 volt
	
	4005
	Decorative cornice

	0028
	8 ounce kettle -240 volt
	
	4006
	Drawer

	1001
	Popper Assembly
	
	4007
	Stainless product tray

	1002
	Chassis Assembly
	
	4008
	Stainless drop door

	1004
	Wiring Diagram
	
	4009
	Canopy Cover

	1011
	50w Warmer element -120 volt
	
	4010
	Left door

	1012
	Warmer clamp
	
	4011
	Right door

	1021
	50w warmer element -240 volt
	
	4012
	Door handle

	1014
	800w kettle element -120 volt
	
	4015
	Door magnet

	1024
	800w kettle element -240 volt
	
	4020
	Door hinge set

	1016
	1000w kettle element -120 volt
	
	4033
	Base - complete

	1026
	1000w kettle element -240 volt
	
	4035
	Terminal box cover

	1018
	1250w kettle element -120 volt
	
	5001
	Left kettle hanger

	1028
	1250w kettle element -240 volt
	
	5002
	Right kettle hanger assembly

	1039
	Thermostat
	
	5003
	Lamp Guard

	2001
	Power switch
	
	5004
	4 ounce kettle lid

	2002 
	Kettle switch
	
	5006
	6 ounce kettle lid

	2005
	Lamp socket
	
	5008
	8 ounce kettle lid

	2006
	Kettle socket
	
	 5009
	Kettle latch

	2007
	Kettle plug
	
	 5014
	4 ounce kettle stirrer

	2008
	Kettle pigtail w/plug
	
	5016
	6 ounce kettle stirrer

	2009
	Power cord w/plug
	
	5018
	8 ounce kettle stirrer

	2018
	120 volt (50/60 hz) gear motor
	
	5021
	Kettle handle complete

	2028
	240 volt (50/60 hz) gear motor
	
	5022
	Kettle handle spring

	3002
	Cart handle
	
	5023
	Kettle hinge-pin/clip

	3003
	Cart wheel
	
	5025
	Kettle lid magnet

	3004
	Cart wheel clips
	
	5026
	Corner Post

	3005
	Cart axle
	
	5029
	Kettle Element Clamp

	3006
	Cart leg – short
	
	5030
	Chassis

	3007
	Cart leg – long
	
	5034
	4 oz. Kettle jacket

	3008
	Cart shelf
	
	5036
	6 oz. Kettle jacket

	3009
	Cart body
	
	5038
	8 oz. Kettle jacket

	3010
	Cart hardware kit
	
	5044
	4 oz. kettle inner

	
	
	
	5046
	6 oz. kettle inner

	
	
	
	5048
	8 oz. kettle inner


To Order Parts Contact:

Benchmark USA, Inc.

25-J Brookfield Oaks Dr.

Greenville, SC 29607 USA

Tel:  877-432-5500
Fax:  864-312-5505

Email:  parts@benchmarkusainc.com
Page 7

[image: image7.png]



Three Year Warranty

THE WARRANTY:

Benchmark USA warrants the original purchaser of new food equipment manufactured by Benchmark USA to be free from defects in material or workmanship  from the date of the original purchase for a period of three years.  

WHO IS COVERED BY THIS WARRANTY:

The original purchaser who has provided proper proof of the original retail purchase and all other requested information.

WHAT IS WARRANTIED:

The obligation of Benchmark USA under this warranty is limited to the replacement of any parts or components that in the opinion of Benchmark USA are defective, F.O.B. the factory, or at any other location that Benchmark USA may designate.

WHAT VOIDS THE WARRANTY:

1.  Any piece of equipment that has been installed, operated or maintained inconsistently 

     with Benchmark USA operating instructions.

2.  Any  part  or  component  that has been modified, changed or altered from its original 

     configuration.

3.  Any   electrical  component   that   has  been   damaged  due  to  submergence  in  any   

     liquid.

4.  Any   part   or   component   that   has   been   subject   to  abuse,  misuse,  neglect  or 

     accidents.

EXTENT OF WARRANTY:

Any defective component, part or assembly returned to Benchmark USA will be replaced by Benchmark USA.  Benchmark USA will not be responsible for any expenses incurred by the customer under the terms of this warranty, nor shall it be responsible for any damages either consequential, special, contingent, or otherwise; or expenses or injury arising directly or indirectly from the use of a Benchmark USA product.  Any component, part or assembly returned to Benchmark USA must be returned at the customers expense along with proof of purchase.  Benchmark USA reserves the right to determine whether the terms of this warranty have been properly complied with.  In the event that the terms are not complied with, Benchmark USA shall be under no obligation to honor this warranty.    

Benchmark USA Inc.  ●  25-J Brookfield Oaks Dr. ●  Greenville, SC 29607 ●  T: 864-312-5500  ●  F: 864-312-5505
�


WARNING ELECTRICAL SHOCK HAZARD.





FAILURE TO FOLLOW THESE INSTRUCTIONS COULD RESULT IN SERIOUS INJURY OR DEATH.








[image: image1]